TECHNICAL DESIGN SPECIFICATION

DX Assistant

Version 1.0 product blueprint and V0.4 implementation baseline

| Document | Value |
| --- | --- |
| Status | Design baseline for review |
| Prepared | 14 July 2026 |
| Current release | V0.4 - receive-only WSJT-X monitor |
| Primary station | Yaesu FTDX101D with WSJT-X / FT8 |

# 1. Project philosophy

Purpose: Determine when a chosen rare DX station is active, decide where to listen, and alert the operator when that station is decoded locally in FT8.

DX Assistant is a receive and decision-support application. It combines reports from external spot sources with local WSJT-X status and decodes. It may tune the receiver only when the operator has enabled that capability; it never initiates transmission.

## Engineering principles

| Principle | Design consequence |
| --- | --- |
| Never transmit | No PTT, WSJT-X reply, message generation, or transmit command is permitted. |
| Operator in control | Monitoring, bands, tuning policy, alarms, and CAT permission are explicit and visible. |
| Known-good releases | Every release has a fixed scope, tests, release notes, and a rollback copy. |
| Radio-quality behaviour | Calm presentation, deterministic states, meaningful alarms, and recoverable errors. |
| DX utility first | A feature belongs only when it materially improves discovery, listening, or response. |

# 2. System architecture

The application is divided into three layers with one-way dependencies. Hardware and network adapters publish observations; the logic engine decides; the interface presents state and accepts operator intent.

| Layer | Responsibilities | V1 interfaces |
| --- | --- | --- |
| Operator interface | Configuration, live status, controls, alarms, history | Desktop dashboard and settings |
| DX logic engine | Target matching, band candidates, ranking, state transitions, alarm policy | Pure Python domain objects |
| Adapters | Receive observations and perform explicitly authorised receiver tuning | WSJT-X UDP, CAT, cluster, PSK Reporter |

## Receive-to-alert flow

| Stage | Input | Required behaviour |
| --- | --- | --- |
| Observe | Cluster / PSK Reporter | Normalise call, frequency, band, reporter, time, and source confidence. |
| Qualify | Target and enabled-band rules | Discard stale, disabled-band, malformed, and duplicate observations. |
| Rank | Operator search policy | Choose strongest evidence, highest enabled band, or manual selection. |
| Listen | CAT permission and WSJT-X state | Tune receiver to the configured FT8 frequency; never alter transmit state. |
| Confirm | Local WSJT-X decode | Match the target as a complete callsign token and record the decode. |
| Alert | Alarm policy | Raise visual and audible alarm until acknowledged; retain evidence in history. |

## Safety boundary

Hard rule: Outbound commands are limited to receiver frequency selection. Any future CAT adapter must reject PTT and transmit-related commands by construction, log every tuning action, and fail closed when radio state is uncertain.

## Data records

Spot observations, WSJT-X status, local decodes, alarms, operator acknowledgements, tuning attempts, and errors are timestamped in UTC. Logs are append-only during a session and use human-readable CSV or JSON Lines formats.

# 3. Operator interface

## Main dashboard

| Area | Displays | Operator controls |
| --- | --- | --- |
| Target | DX callsign, expedition label, monitoring state | Target callsign; Start; Stop |
| Station | WSJT-X link, CAT link, radio frequency, mode, last heartbeat | Reconnect adapters; open diagnostics |
| Bands | 40/30/20/17/15/12/10 m enabled state, configured frequency, last spot, evidence | Enable band; edit frequency; listen now |
| Search | Current policy, chosen band, reason for choice, dwell timer | Strongest evidence; highest band; manual |
| Local decodes | Time, SNR, audio offset, message, target highlight | Filter; copy; open log |
| Alarm | Target, band, frequency, SNR, first/last decode, count | Acknowledge; Mute; Keep monitoring |

## Settings screen

| Group | Settings |
| --- | --- |
| WSJT-X | UDP address, port, expected station ID, heartbeat timeout |
| Radio / CAT | Disabled by default; model, port, baud, poll interval, tune permission |
| Bands | Enabled switch and FT8 receive frequency for each supported band |
| Spot sources | Cluster endpoint, PSK Reporter query interval, source enable switches |
| Search policy | Strongest evidence, highest band, manual; dwell and evidence expiry |
| Alarm and logs | Sound, repeat interval, mute duration, log directory, retention |

## Control behaviour

Start validates the configuration and enters monitoring only if WSJT-X can be heard. Stop cancels source polling and automatic tuning but leaves the screen and logs available. Acknowledge silences the current alarm without deleting it. Mute suppresses sound for a visible, time-limited interval; visual alarms remain. Listen now is enabled only on an operator-enabled band and never transmits.

# 4. States, alarms and failure behaviour

| State | Meaning | Exit condition |
| --- | --- | --- |
| Stopped | No monitoring or automatic tuning | Operator selects Start |
| Starting | Configuration and adapter checks in progress | Checks pass or an error is raised |
| Monitoring | Sources and WSJT-X active; no qualified target evidence | Qualified spot, stop, degradation, or error |
| Target reported | Fresh target evidence exists on one or more enabled bands | Evidence expires or a candidate is selected |
| Searching | Receiver is listening on the selected candidate frequency | Target decoded, dwell expires, stop, or failure |
| Target decoded | The target was decoded locally | Operator acknowledges; monitoring may continue |
| Degraded | One non-critical source or adapter is unavailable | Recovery, stop, or escalation to error |
| Error | Safe monitoring cannot continue | Operator resolves condition and restarts |

## Alarm catalogue

| Alarm | Priority | Presentation and acknowledgement |
| --- | --- | --- |
| Target decoded locally | Critical DX | Distinct sound, persistent red/amber banner, taskbar attention; operator acknowledgement required |
| Target reported | Information | Quiet banner and band highlight; no sound by default |
| WSJT-X heartbeat lost | Warning | Amber status and single chime; automatic recovery when heartbeat returns |
| CAT mismatch or tune failure | Warning | No repeated tuning; show requested and actual frequency; acknowledgement required |
| Configuration invalid | Blocking | Start refused; identify the exact field and preserve entered values |
| Log write failure | Warning | Visible persistent warning; monitoring may continue with in-memory buffer |

## Failure policy

Malformed packets and stale spots are ignored and counted. Loss of an external spot source causes Degraded state, not uncontrolled retuning. Loss of WSJT-X or uncertain CAT state stops automatic tuning. Restart never resumes an unacknowledged tuning action from the previous session.

# 5. Configuration and release roadmap

## Configuration contract

| Item | Default | Validation |
| --- | --- | --- |
| Target callsign | Required | One normalised callsign; no whitespace |
| WSJT-X UDP | 127.0.0.1:2237 | Valid address and port; bind must succeed |
| Bands | 40-10 m enabled | Supported band; positive receive frequency |
| CAT tune permission | Off | Explicit opt-in; adapter must identify the FTDX101D |
| Search policy | Highest enabled band | One of strongest, highest, or manual |
| Evidence lifetime | 10 minutes | 1-60 minutes |
| Dwell time | 2 minutes | 15 seconds-10 minutes |
| Alarm sound | On | Selectable sound; test control available |
| Logging | Daily CSV | Writable folder; UTC timestamps |

## Roadmap

| Release | Deliverable | Completion evidence |
| --- | --- | --- |
| V0.4 | Layered core, validated configuration, heartbeat/status/decode parsing, target alarm, daily decode log | Automated packet, engine, logging, and safety tests |
| V0.5 | Desktop dashboard and explicit application state machine | Operator walkthrough with simulated WSJT-X traffic |
| V0.6 | Read-only FTDX101D CAT status, then separately gated receiver tuning | Bench test proving no PTT/transmit commands |
| V0.7 | Cluster adapter and normalised spot evidence | Recorded-source replay and stale/duplicate filtering tests |
| V0.8 | PSK Reporter adapter and multi-source confidence | Rate-limit, outage, and evidence-merging tests |
| V0.9 | Band ranking, dwell/search logic, alarms, persistence, installer candidate | Full station rehearsal and recovery tests |
| V1.0 | Known-good operator release | Acceptance checklist, signed release notes, rollback package |

## V1.0 acceptance conditions

The release must run for an eight-hour monitoring session without data loss; recover cleanly from WSJT-X, CAT, and internet-source interruption; identify a target decode without partial-callsign false positives; record every state-changing action; and demonstrate by code inspection and bench test that it cannot initiate transmission.

## Deliberately deferred to Version 2

Propagation prediction, map visualisation, multiple simultaneous targets, award/logbook integration, mobile notifications, collaborative spotting, and automated expedition schedule ingestion remain future ideas. None should delay a safe and reliable Version 1.0.